## Concept Note : AI + FEA–Driven Generative Design of an Endodontic Rotary File System

Clinical Partner: Dr. Shrey Dhawan (MDS Endodontics)

### 1. Clinical Problem & Opportunity

Modern endodontic treatment relies heavily on NiTi rotary file systems to clean and shape root canals. However:

- Root canals have highly variable 3D anatomies (curvatures, diameters, S-shapes, abrupt apical curves).

- Current file systems are largely designed by trial-and-error and empirical experience, not by systematic simulation.

- Failures such as cyclic fatigue fracture, torsional failure, canal transportation, ledging and micro-cracks are still common.

- There is no transparent, simulation-backed method to justify why one file design is “safer” or “more conservative” than another.

This presents a clear opportunity to build a science-first, simulation-backed file system, combining Finite Element Analysis (FEA) and AI-driven optimization.

### 2. Proposed Idea in One Line

Develop a generative design framework using FEA + AI to create a new rotary NiTi file system that is optimized for real-world canal anatomies, with quantified safety margins and performance metrics.

### 3. Technical Approach (High-Level)

a) Parametric Geometry Definition

Create a parametric CAD model of a rotary file with control over key design variables:

- Cross-sectional shape (triangular, rectangular, S-shape, etc.)

- Taper (constant / variable) and tip size

- Helix angle, pitch, variable core diameter

- Rake angle and flute depth

- File length and working segment design

This forms the design parameter space.

b) FEA Workflow

Build an FEA pipeline to evaluate each candidate design under clinically realistic conditions:

- Material model: Superelastic NiTi (with appropriate stress–strain and phase transformation behavior).

- Load cases:

  - Cyclic flexural fatigue in curved canals (different radii and arc lengths).

  - Torsional loading with tip locked (simulating binding).

- Outputs:

  - Peak von Mises stress distribution.

  - Safety factors for cyclic fatigue and torsional failure.

  - Canal transportation / straightening tendencies (via coupled geometrical analysis).

Goal: create a simulation dataset mapping design parameters → performance metrics.

c) AI / Machine Learning Layer

Use the FEA-generated dataset to train surrogate models:

- Input: design parameters (geometry, taper, etc.)

- Output: predicted performance metrics (stress, fatigue life, safety factors, etc.)

Potential approaches:

- Gradient-boosted trees / random forests for tabular param data.

- Neural networks or physics-informed models if needed for complex interactions.

Once the surrogate is reliable, we can perform rapid multi-objective optimization to:

- Minimize peak stress and strain.

- Maximize fatigue life and safety factor.

- Constrain for clinically acceptable flexibility, taper, and cutting efficiency.

This yields a Pareto front of “best possible” designs rather than a single arbitrary geometry.

d) From Design Space to File System

From the optimized solutions, we then:

- Cluster designs into a clinically usable sequence (e.g., 3–5 files) with logical progression of tip sizes and tapers.

- Match each file’s design to typical canal anatomies (moderate curvature, severe curvature, retreatment files, etc.).

- Prepare candidates for bench-top testing (ISO-based fatigue tests, torsion tests) and later in-vitro / in-vivo validation.

### 4. Expected Outcomes

- Simulation-backed file system with transparent design rationale.

- A reusable FEA + AI design platform that can be adapted to:

  - Different NiTi alloys / heat treatments.

  - Different file families (or even other dental tools).

- Potential for joint patents, publications, and industrial partnerships with file manufacturers in India and abroad.